如果想实现torch模型实时推理相机视频流，请先去看配置torch的教程

建议在虚拟环境里进行以下教程，因为安装了很多包，怕污染主环境

1. 硬件连接

网线直连：用A8 mini附带的JST-GH转RJ45线连接相机，RJ45端插入Jetson Orin Nano的网口。

独立供电：用电源线给A8 mini供电（11-25.2V，3-6S电池）。同时给Jetson Orin Nano通电。

2. 软件配置

修改Jetson网络：需要将Jetson的IP设为和A8 mini同一网段，因为A8 mini出厂默认IP是192.168.144.25。

图形界面：在Ubuntu设置里为有线连接手动配置一个IP，比如192.168.144.100。

验证连接：

首先安装arp-scan

bash

sudo apt update && sudo apt install arp-scan -y

然后用arp-scan扫描当前子网，看看相机是否正常工作以及相机的IP

bash

sudo arp-scan --interface=enP8p1s0 192.168.144.0/24

预期输出

Interface: enP8p1s0, type: EN10MB, MAC: 3c:6d:66:be:bb:88, IPv4: 192.168.144.100 Starting arp-scan 1.9.7 with 256 hosts (https://github.com/royhills/arp-scan) 192.168.144.25 7c:d1:37:a8:20:22 (Unknown) 1 packets received by filter, 0 packets dropped by kernel Ending arp-scan 1.9.7: 256 hosts scanned in 2.042 seconds (125.37 hosts/sec). 1 responded

黄色部分证明相机IP确实是192.168.144.25

获取视频流：A8 mini默认通过RTSP协议输出视频流。

先检查 RTSP 端口是否开放

bash

sudo apt install nmap -y nmap -p 8554 192.168.144.25

预期输出

bash

Starting Nmap 7.80 ( https://nmap.org ) at 2026-04-20 21:47 CST Nmap scan report for 192.168.144.25 Host is up (0.00037s latency). PORT STATE SERVICE 8554/tcp open rtsp-alt Nmap done: 1 IP address (1 host up) scanned in 0.06 seconds

使用 ffplay 测试相机是否能输出视频流

bash

sudo apt install ffmpeg -y ffplay rtsp://192.168.144.25:8554/main.264 -fflags nobuffer -flags low_delay -framedrop

预期会输出相机实施拍摄的画面

3 python脚本（OpemCV+FFmpeg）（减少视频采集缓冲区+异步双线程 （Pipeline）降低延迟）

创建python脚本drone_inference.py

python

import cv2 import torch import time import argparse import os import threading import queue from pathlib import Path from ultralytics import YOLO class LowLatencyYOLOPipeline: def __init__(self, model_path, video_source, width=640, height=480, fps=30): # 优化 RTSP 传输参数：TCP、低延迟标志、减小缓冲 os.environ['OPENCV_FFMPEG_CAPTURE_OPTIONS'] = ( 'rtsp_transport;tcp¦max_delay;100000¦buffer_size;512000¦flags;low_delay' ) # 加载模型（建议使用 TensorRT 引擎文件 .engine 以获得最低延迟） self.model = self.load_model(model_path) self.video_source = video_source self.use_rtsp = isinstance(video_source, str) and video_source.startswith('rtsp://') self.width = width self.height = height self.fps = fps # 打开视频源并设置缓冲区大小为 1（关键优化） self.cap = self._open_capture() if not self.cap.isOpened(): raise RuntimeError(f"无法打开视频源: {video_source}") # 读取实际参数 self.frame_width = int(self.cap.get(cv2.CAP_PROP_FRAME_WIDTH)) self.frame_height = int(self.cap.get(cv2.CAP_PROP_FRAME_HEIGHT)) self.frame_fps = self.cap.get(cv2.CAP_PROP_FPS) print(f"视频源已开启: {self.frame_width}x{self.frame_height} @ {self.frame_fps:.2f}fps") print(f"模型加载成功: {Path(model_path).name}") # 异步队列：最多缓存 2 帧，丢弃旧帧以保持最低延迟 self.frame_queue = queue.Queue(maxsize=2) self.running = True self.reader_thread = threading.Thread(target=self._reader_worker, daemon=True) def _open_capture(self): """打开视频捕获设备，强制设置缓冲区大小为 1""" if isinstance(self.video_source, int) or (isinstance(self.video_source, str) and self.video_source.isdigit()): source = int(self.video_source) cap = cv2.VideoCapture(source, cv2.CAP_V4L2) cap.set(cv2.CAP_PROP_FRAME_WIDTH, self.width) cap.set(cv2.CAP_PROP_FRAME_HEIGHT, self.height) cap.set(cv2.CAP_PROP_FPS, self.fps) else: # RTSP 或视频文件，强制使用 FFMPEG cap = cv2.VideoCapture(self.video_source, cv2.CAP_FFMPEG) # 关键：将缓冲区大小设为 1，避免 OpenCV 内部缓存多帧引入延迟 cap.set(cv2.CAP_PROP_BUFFERSIZE, 1) return cap def load_model(self, model_path): model = YOLO(model_path) if torch.cuda.is_available(): model.to('cuda') return model def _reader_worker(self): """生产者线程：持续读取帧，只保留最新的一帧""" reconnect_attempts = 0 while self.running: ret, frame = self.cap.read() if not ret: print("读取帧失败，尝试重新连接...") self.cap.release() time.sleep(1) # 重新打开 try: self.cap = self._open_capture() if self.cap.isOpened(): reconnect_attempts = 0 print("重连成功") continue except Exception as e: print(f"重连异常: {e}") reconnect_attempts += 1 if reconnect_attempts > 5: print("重连失败超过5次，退出程序") self.running = False break continue reconnect_attempts = 0 # 成功读取后重置计数 # 队列若已满，则丢弃最旧的帧（保持最新帧） if self.frame_queue.full(): try: self.frame_queue.get_nowait() except queue.Empty: pass self.frame_queue.put(frame) def infer_frame(self, frame): results = self.model(frame, verbose=False) result = results[0] infer_time = result.speed['inference'] / 1000.0 return result, infer_time def draw_results(self, frame, result, infer_time): if result.boxes is not None: for box in result.boxes: x1, y1, x2, y2 = map(int, box.xyxy[0].tolist()) conf = float(box.conf[0]) cls_id = int(box.cls[0]) label = f"{result.names[cls_id]} {conf:.2f}" cv2.rectangle(frame, (x1, y1), (x2, y2), (0, 255, 0), 2) cv2.putText(frame, label, (x1, y1-10), cv2.FONT_HERSHEY_SIMPLEX, 0.5, (0, 255, 0), 2) fps_text = f"FPS: {1/infer_time:.1f}" if infer_time > 0 else "FPS: --" cv2.putText(frame, fps_text, (10, 30), cv2.FONT_HERSHEY_SIMPLEX, 1, (0, 0, 255), 2) cv2.putText(frame, f"Infer: {infer_time*1000:.1f}ms", (10, 60), cv2.FONT_HERSHEY_SIMPLEX, 0.7, (0, 0, 255), 2) return frame def run(self): """消费者主线程：从队列获取最新帧，进行推理并显示""" self.reader_thread.start() print("开始推理（低延迟模式），按 'q' 键退出...") while self.running: try: # 等待新帧，超时 1 秒 frame = self.frame_queue.get(timeout=1.0) except queue.Empty: if not self.running: break continue # 推理与绘制 result, infer_time = self.infer_frame(frame) display_frame = self.draw_results(frame.copy(), result, infer_time) cv2.imshow('YOLO Detection (Low Latency)', display_frame) # 按 'q' 退出 if cv2.waitKey(1) & 0xFF == ord('q'): self.running = False break # 清理资源 self.running = False self.reader_thread.join(timeout=1.0) self.cap.release() cv2.destroyAllWindows() def main(): parser = argparse.ArgumentParser(description="YOLO 推理管道") parser.add_argument('--model', type=str, required=True, help='模型路径 (.pt 或 .engine)') parser.add_argument('--source', type=str, default='0', help='视频源：摄像头ID、RTSP地址或视频文件') parser.add_argument('--width', type=int, default=640, help='采集宽度') parser.add_argument('--height', type=int, default=480, help='采集高度') parser.add_argument('--fps', type=int, default=30, help='采集帧率') args = parser.parse_args() if not Path(args.model).exists(): print(f"错误：模型文件不存在 {args.model}") return pipeline = LowLatencyYOLOPipeline( model_path=args.model, video_source=args.source, width=args.width, height=args.height, fps=args.fps ) try: pipeline.run() except KeyboardInterrupt: print("\n程序被用户中断") except Exception as e: print(f"运行时错误: {e}") if __name__ == "__main__": main()

运行脚本

bash

python drone_inference.py --model /home/nvidia/CAMERA/best.pt --source "rtsp://192.168.144.25:8554/main.264"